BUILDING A SALAD RECIRCULATING PUMP

Tools Needed:

Drill or Drill Press

⅛” Drill Bit

1 ¼” Drill Bit / Hole Saw

Channel Locks

Razor Knife

Belt Sander

Time Involved : 5 Minutes

Recommended Skills: Safe use of listed tools

Step #1

- Remove the cap from suction canister lid

- There is a letter “R” in the center of the lid on the inside. The letter “R” is in the center of the cap

- Drill a ⅛” pilot hole

Step #2

- Flip the cap over and

- Grasp the cap with channel locks

- Drill a 1 ¼” hole

Step #3

- Completely disassemble the fluid pump

Step #4

- Note that the diameter of the pump base is larger than the inside diameter of the cap

- Gently sand the circumference of the white lip of the pump until it can fit inside the suction canister cap

Step #5

- Reassemble the pump

Step #6

- Note that the pickup tube is too large to fit the depth of a 1200cc suction canister

- Cut the pickup tube to the desired length. Approximately 2 ½”

Step #7

- Train with your new set up